S5735I-S16T2S4XN-V2

Industrial Ethernet Switch

16 x GE RJ45 | 2 x GE SFP | 4 x 10GE SFP+ | DIN Rail | Fanless | Dual DC Power

Part Number: 98012427

Hi-Network.com

Datasheet V1.0 | May 2026

Table of Contents

Right-click the TOC and select "Update Field" to refresh page numbers

Table of Contents	1

Product Overview	1

Product Highlights	1

Ports and Interfaces	1

Ethernet Ports	1

Optical Ports	1

Management Ports	1

Industrial I/O Ports	1

Technical Specifications	1

Physical Specifications	1

Power Specifications	1

Environmental Specifications	1

Reliability Specifications	1

Compliance and Certification	1

Ordering Information	1

Product Overview

The S5735I-S16T2S4XN-V2 is a high-performance industrial Ethernet switch designed for harsh environments. Featuring 16 Gigabit Ethernet ports, 2 GE SFP optical ports, and 4 10GE SFP+ uplink ports, this fanless switch delivers reliable Layer 2 switching with industrial-grade durability.

With DIN rail mounting, dual redundant DC power input (9.6V to 60V), and natural heat dissipation, the S5735I-S16T2S4XN-V2 is ideal for industrial automation, smart transportation, power utilities, and other demanding deployment scenarios.

| Description | S5735I-S16T2S4XN-V2 (16 x 10/100/1000BASE-T ports, 2 x GE SFP ports, 4 x 10GE SFP+ ports, DIN Rail Mounting, Dual redundant 9.6 to 60V DC power, Fanless) |
| --- | --- |
| Part Number | 98012427 |
| First Supported Version | V600R024C00 |
| Switching Capacity | 128 Gbps |
| Packet Forwarding Rate | 96 Mpps |
| Mounting | DIN Rail |
| Cooling | Natural heat dissipation (fanless) |

Product Highlights

- Industrial-grade Design —IP40 ingress protection, metal chassis, and fanless operation for reliable performance in harsh environments.

- Flexible Port Configuration —16 x GE RJ45 ports + 2 x GE SFP + 4 x 10GE SFP+ with software-configurable port combination modes (M0/M1).

- Wide-range Power Input —Dual redundant DC power input from 9.6V to 60V DC supports various industrial power scenarios.

- Extended Temperature Range —Operates from -40 degrees C to +75 degrees C depending on cabinet ventilation configuration.

- Rich Industrial Interfaces —Built-in RS-485, DI/DO ports, and USB for industrial IoT and automation applications.

- High Reliability —1+1 power redundancy, 62.87-year MTBF, and > 0.99999 availability.

Ports and Interfaces

Ethernet Ports

The switch provides 16 x 10/100/1000BASE-T RJ45 ports organized in two groups of 8 ports each. Ports 1-8 and 9-16 support auto-negotiation for 10 Mbps, 100 Mbps, or 1000 Mbps operation.

| Port Type | 16 x 10/100/1000BASE-T RJ45 |
| --- | --- |
| Speed | 10/100/1000 Mbps auto-negotiation |
| Cable Type | Cat5e/Cat6 UTP/STP |
| Max Distance | 100 meters |

Optical Ports

The switch provides 2 x GE SFP ports and 4 x 10GE SFP+ ports for fiber uplink connectivity. Port combination modes allow flexible allocation between electrical and optical interfaces.

| Port Type | 2 x GE SFP + 4 x 10GE SFP+ |
| --- | --- |
| SFP Ports | 1000BASE-X (1 Gbps) |
| SFP+ Ports | 10GBASE-X (10 Gbps) |
| Combo Mode M0 (Default) | 8 x GE RJ45 + 2 x GE SFP (ports 5-6) |
| Combo Mode M1 | 4 x GE RJ45 + 2 x 10GE SFP+ (ports 5-6) |

Note: In M1 mode, GE electrical ports 13-16 are unavailable. 10GE SFP+ modules cannot be installed in SFP ports 5 and 6 in M0 mode.

Management Ports

| Console Port | 1 x RJ45 console port |
| --- | --- |
| USB Port | 1 x USB 2.0 Type A (supports USB flash drive for configuration and upgrades) |

Industrial I/O Ports

The switch includes industrial-grade I/O interfaces for integration with automation systems:

| RS-485 Port | 2x5-Pin Phoenix Connector, half-duplex |
| --- | --- |
| Baud Rates | 1200 / 2400 / 4800 / 9600 / 19200 / 38400 / 57600 / 115200 bit/s |
| Max Nodes | 64 |
| Max Distance | 1 km at 9600 bit/s |
| DI Port | 2x5-Pin Phoenix Connector, passive DI signals |
| DO Port | 2x5-Pin Phoenix Connector, passive DO signals (External power < 60V DC, < 0.5A) |

Technical Specifications

Physical Specifications

| Dimensions (H x W x D) | 150.0 x 127.0 x 133.0 mm (5.91 x 5.00 x 5.24 in.) |
| --- | --- |
| Max Dimensions | 162.0 x 127.0 x 142.0 mm (6.38 x 5.00 x 5.59 in.) |
| Package Dimensions | 175.0 x 230.0 x 280.0 mm (6.89 x 9.06 x 11.02 in.) |
| Chassis Height | 3.65 U |
| Chassis Material | Metal |
| Weight (without packaging) | 2.527 kg (5.57 lb) |
| Weight (with packaging) | 3.019 kg (6.66 lb) |
| Mounting | DIN Rail |
| IP Rating | IP40 |

Power Specifications

| Power Supply Mode | DC external |
| --- | --- |
| Rated Input Voltage | 12V DC to 48V DC |
| Input Voltage Range | 9.6V DC to 60V DC |
| Maximum Input Current | 3 A |
| Redundant Power | 1+1 redundancy |
| Typical Power Consumption | 22.3 W |
| Maximum Power Consumption | 31.3 W at 10.8V / 27.5 W at 48V |
| Typical Heat Dissipation | 76.09 BTU/hour |
| Maximum Heat Dissipation | 106.80 BTU/hour at 10.8V / 93.83 BTU/hour at 48V |
| Static Power Consumption | 14.5 W |
| Power Surge Protection | +/- 2 kV common mode, +/- 1 kV differential mode |

Environmental Specifications

| Operating Temperature (Industrial Optics) | -40 degrees C to +60 degrees C (sealed cabinet) / -40 degrees C to +70 degrees C (vented cabinet >= 80 LFM) / -40 degrees C to +75 degrees C (blower cabinet >= 200 LFM) |
| --- | --- |
| Operating Temperature (PEN Optics) | 0 degrees C to +45 degrees C (sealed cabinet) / 0 degrees C to +55 degrees C (vented cabinet) / 0 degrees C to +60 degrees C (blower cabinet) |
| Storage Temperature | -40 degrees C to +85 degrees C (-40 degrees F to +185 degrees F) |
| Operating Humidity | 5% RH to 95% RH, non-condensing |
| Operating Altitude | 0 - 5000 m (0 - 16404 ft.) |
| Storage Altitude | 0 - 5000 m (0 - 16404 ft.) |
| Altitude Derating | Above 1800 m: decrease 1 degrees C per 220 m increase |

Note: When ambient temperature exceeds 45 degrees C, USB and RS-485 ports are not recommended for use. When using low-voltage input in sealed cabinets, optical module installation restrictions apply based on voltage and temperature conditions.

Reliability Specifications

| MTBF | 62.87 years |
| --- | --- |
| Availability | > 0.99999 |
| Noise Level | Noise-free (fanless design) |
| Service Port Surge Protection | +/- 6 kV common mode |
| Memory | 2 GB |
| Flash Memory | 1 GB |

Compliance and Certification

| EMC Certification | Compliant |
| --- | --- |
| Safety Certification | Compliant |
| Manufacturing Certification | Compliant |
| Ingress Protection | IP40 (dustproof/waterproof) |

Ordering Information

To place an order, please contact Hi-Network.com or your authorized reseller.

| Model | S5735I-S16T2S4XN-V2 |
| --- | --- |
| Part Number | 98012427 |
| Description | 16 x 10/100/1000BASE-T ports, 2 x GE SFP ports, 4 x 10GE SFP+ ports, DIN Rail Mounting, Dual redundant 9.6 to 60V DC power, Fanless |

For more information, please visit: https://www.Hi-Network.com

Hi-Network.com

Your Trusted Network Equipment Partner

www.Hi-Network.com

info@hi-network.com