S5735I-S8T4SN-V2

Industrial Ethernet Switch

8 x GE RJ45 | 4 x GE SFP | DIN Rail | Fanless | Dual DC Power

Part Number: 98012086

Hi-Network.com

Datasheet V1.0 | May 2026

Table of Contents

Right-click the TOC and select "Update Field" to refresh page numbers

Product Overview	3

Product Highlights	4

Ports and Interfaces	5

Ethernet Ports	5

Optical Ports	5

Console Port	6

Industrial I/O Ports	6

Technical Specifications	7

Physical Specifications	7

Power Specifications	7

Environmental Specifications	8

Reliability Specifications	8

Compliance and Certification	9

Ordering Information	10

Product Overview

The S5735I-S8T4SN-V2 is a compact industrial Ethernet switch designed for space-constrained harsh environments. Featuring 8 Gigabit Ethernet ports and 4 GE SFP optical ports, this fanless switch delivers reliable Layer 2 switching in a slim, lightweight form factor.

With DIN rail mounting, dual redundant DC power input (9.6V to 60V), and natural heat dissipation, the S5735I-S8T4SN-V2 is ideal for industrial automation, smart transportation, power utilities, and other demanding deployment scenarios where space is limited.

| Description | S5735I-S8T4SN-V2 (8 x 10/100/1000BASE-T ports, 4 x GE SFP ports, DIN Rail Mounting, Dual redundant 9.6 to 60V DC power, Fanless) |
| --- | --- |
| Part Number | 98012086 |
| First Supported Version | V600R022C01 |
| Switching Capacity | 36 Gbps |
| Packet Forwarding Rate | 18 Mpps |
| Mounting | DIN Rail |
| Cooling | Natural heat dissipation (fanless) |

Product Highlights

- Ultra-compact Design —Only 46mm height (3.65U) and 0.95kg weight, ideal for space-constrained installations.

- Industrial-grade Durability —IP40 ingress protection, metal chassis, and fanless operation for reliable performance in harsh environments.

- Flexible Uplink Options —8 x GE RJ45 ports + 4 x GE SFP optical ports support fiber connectivity up to 10 km.

- Wide-range Power Input —Dual redundant DC power input from 9.6V to 60V DC supports various industrial power scenarios.

- Extended Temperature Range —Operates from -40 degrees C to +75 degrees C depending on cabinet ventilation configuration.

- Rich Industrial Interfaces —Built-in RS-485, DI/DO ports for industrial IoT and automation applications.

- Exceptional Reliability —1+1 power redundancy, 104.86-year MTBF, and > 0.99999 availability.

Ports and Interfaces

Ethernet Ports

The switch provides 8 x 10/100/1000BASE-T RJ45 ports supporting auto-negotiation for 10 Mbps, 100 Mbps, or 1000 Mbps operation.

| Port Type | 8 x 10/100/1000BASE-T RJ45 |
| --- | --- |
| Speed | 10/100/1000 Mbps auto-negotiation |
| Cable Type | Cat5e/Cat6 UTP/STP |
| Max Distance | 100 meters |

Optical Ports

The switch provides 4 x 1000BASE-X SFP ports for fiber connectivity. These ports support industrial optical modules with 1 Gbps transmission speed and distances up to 10 km. In V600R022C10 and later, SFP+ high-speed copper cables and dedicated stack cables are supported for stack connections.

| Port Type | 4 x 1000BASE-X SFP |
| --- | --- |
| Speed | 1000 Mbps |
| Max Fiber Distance | Up to 10 km (depending on module) |
| Stacking Support | SFP+ high-speed copper cables (3m) and dedicated stack cables (0.5m/1.5m) from V600R022C10 |

Console Port

The switch provides one RJ45 console port for local management and configuration.

Industrial I/O Ports

The switch includes industrial-grade I/O interfaces for integration with automation systems:

| RS-485 Port | Half-duplex, 2x5-Pin Phoenix Connector |
| --- | --- |
| Baud Rates | 1200 / 2400 / 4800 / 9600 / 19200 / 38400 / 57600 / 115200 bit/s |
| Default Baud Rate | 9600 bit/s (8 data bits, 1 stop bit, no parity, no flow control) |
| Max Nodes | 64 |
| Max Distance | 1 km at 9600 bit/s |
| DI Port | 2x5-Pin Phoenix Connector, passive DI signals, Boolean value |
| DO Port | 2x5-Pin Phoenix Connector, passive DO signals (External power < 60V DC, < 0.5A) |

Technical Specifications

Physical Specifications

| Dimensions (H x W x D) | 150.0 x 46.0 x 133.0 mm (5.91 x 1.81 x 5.24 in.) |
| --- | --- |
| Max Dimensions | 162.0 x 46.0 x 142.0 mm (6.38 x 1.81 x 5.59 in.) |
| Package Dimensions | 235.0 x 90.0 x 205.0 mm (9.25 x 3.54 x 8.07 in.) |
| Chassis Height | 3.65 U |
| Chassis Material | Metal |
| Weight (without packaging) | 0.95 kg (2.09 lb) |
| Weight (with packaging) | 1.23 kg (2.71 lb) |
| Mounting | DIN Rail |
| IP Rating | IP40 |

Power Specifications

| Power Supply Mode | DC external |
| --- | --- |
| Rated Input Voltage | 12V DC to 48V DC |
| Input Voltage Range | 9.6V DC to 60V DC |
| Maximum Input Current | 2 A |
| Redundant Power | 1+1 redundancy |
| Typical Power Consumption | 17.44 W |
| Maximum Power Consumption | 18.59 W |
| Typical Heat Dissipation | 59.48 BTU/hour |
| Maximum Heat Dissipation | 63.40 BTU/hour |
| Static Power Consumption | 14.10 W |
| Power Surge Protection | +/- 2 kV common mode, +/- 1 kV differential mode |

Environmental Specifications

| Operating Temperature (Industrial Optics) | -40 degrees C to +60 degrees C (sealed cabinet) / -40 degrees C to +70 degrees C (vented cabinet >= 80 LFM) / -40 degrees C to +75 degrees C (blower cabinet >= 200 LFM) |
| --- | --- |
| Operating Temperature (PEN Optics) | 0 degrees C to +45 degrees C (sealed cabinet) / 0 degrees C to +55 degrees C (vented cabinet) / 0 degrees C to +60 degrees C (blower cabinet) |
| Storage Temperature | -40 degrees C to +85 degrees C (-40 degrees F to +185 degrees F) |
| Operating Humidity | 5% RH to 95% RH, non-condensing |
| Operating Altitude | 0 - 5000 m (0 - 16404 ft.) |
| Storage Altitude | 0 - 5000 m (0 - 16404 ft.) |
| Altitude Derating | Above 1800 m: decrease 1 degrees C per 220 m increase |

Note: When the altitude is between 1800-5000 m (5906-16404 ft.), the highest operating temperature reduces by 1 degrees C (1.8 degrees F) every time the altitude increases by 220 m (722 ft.).

Reliability Specifications

| MTBF | 104.86 years |
| --- | --- |
| Availability | > 0.99999 |
| Noise Level | Noise-free (fanless design) |
| Service Port Surge Protection | +/- 7 kV common mode |
| Memory | 2 GB |
| Flash Memory | 1 GB |

Compliance and Certification

| EMC Certification | Compliant |
| --- | --- |
| Safety Certification | Compliant |
| Manufacturing Certification | Compliant |
| Ingress Protection | IP40 (dustproof/waterproof) |

Ordering Information

To place an order, please contact Hi-Network.com or your authorized reseller.

| Model | S5735I-S8T4SN-V2 |
| --- | --- |
| Part Number | 98012086 |
| Description | 8 x 10/100/1000BASE-T ports, 4 x GE SFP ports, DIN Rail Mounting, Dual redundant 9.6 to 60V DC power, Fanless |

For more information, please visit: https://www.Hi-Network.com

Hi-Network.com

Your Trusted Network Equipment Partner

www.Hi-Network.com

info@hi-network.com