S5735I-H8U2XN-V2

Industrial Ethernet Switch

8 x 10/100/1000BASE-T PoE++ | 2 x 10GE SFP+ | DIN Rail | Fanless | IP40

Part Number: 98012685

Hi-Network.com  |  info@hi-network.com

# Table of Contents

Right-click and select "Update Field" to refresh page numbers

Product Overview	3

Port Configuration	4

Indicators and Buttons	5

Power Supply System	6

Technical Specifications	7

Physical and Environmental	7

Performance and Capacity	8

Ordering Information	9

# Product Overview

The S5735I-H8U2XN-V2 is a high-performance industrial Ethernet switch designed for harsh environments. It features 8 x 10/100/1000BASE-T PoE++ ports and 2 x 10GE SFP+ uplink ports, providing robust connectivity for industrial IoT, smart manufacturing, and outdoor deployments.

With its fanless design, IP40 ingress protection, and wide operating temperature range of -40°C to +75°C, this switch delivers enterprise-grade reliability in demanding industrial scenarios. The DIN rail mounting and dual redundant DC power supply ensure flexible deployment and high availability.

## Key Features

•  8 x 10/100/1000BASE-T PoE++ ports with up to 90W per port

•  2 x 10GE SFP+ uplink ports for high-bandwidth aggregation

•  Fanless design with natural heat dissipation for silent operation

•  IP40 ingress protection against dust and particles

•  DIN rail mounting for industrial cabinet installation

•  Dual redundant 54-57V DC power supply with 1+1 redundancy

•  IEEE 1588v2 precision time protocol support

•  Wide operating temperature: -40°C to +75°C

## Product Specifications at a Glance

| Part Number | 98012685 |
| --- | --- |
| Model | S5735I-H8U2XN-V2 |
| First Supported Version | V600R024C10 |
| Switching Capacity | 56 Gbps |
| Packet Forwarding Rate | 41.66 Mpps |
| MAC Address Table | 16K |
| Dimensions (H x W x D) | 150 x 44 x 133 mm |
| Weight | 1 kg |
| MTBF | 67.24 years |
| Availability | > 0.99999 |

# Port Configuration

The S5735I-H8U2XN-V2 provides a versatile combination of PoE++ access ports and 10GE SFP+ uplink ports to meet diverse industrial connectivity requirements.

## Port Details

| Port | Connector | Description | Quantity |
| --- | --- | --- | --- |
| 10/100/1000BASE-T | RJ45 | PoE++ Ethernet port, 10/100/1000 Mbps | 8 |
| 10GE SFP+ | SFP+ | 10GbE optical uplink, auto-sensing to 1G | 2 |
| Console | RJ45 | Console port for on-site configuration | 1 |
| USB | USB 2.0 Type A | USB flash drive for upgrade and file transfer | 1 |
| DI | 2x5-Pin Phoenix | Digital input, 9.6V-30V state 1 | 1 |
| DO | 2x5-Pin Phoenix | Digital output, passive DO signals | 1 |
| DC Power Input | 5-Pin Terminal | Dual DC power input (DC1/DC2) | 1 |

## PoE Power Specifications

The switch supports PoE, PoE+, and PoE++ (IEEE 802.3af/at/bt) with flexible power configurations:

| PoE Standard | IEEE 802.3af / 802.3at / 802.3bt |
| --- | --- |
| Max Port Power (802.3bt) | 90W per port |
| Max Available PoE Power | Up to 240W |
| Input Voltage for PoE++ | 54-57V DC |
| Input Voltage for PoE | 48V DC |

# Indicators and Buttons

The switch provides comprehensive LED indicators for system monitoring and troubleshooting.

## System Indicators

| Indicator | Name | Description |
| --- | --- | --- |
| SYS | System Status | Green slow blinking = normal; Red = fault |
| PWR1/PWR2 | Power Status | Green = power normal; Off = no power/fault |
| MST | Stack Status | Green blinking = master; Green steady = standby |
| PoE | PoE Mode | Green = PoE mode active |
| USB | USB Status | Green = deployment success; Blinking = reading |
| 1-8 | Port LEDs | Green = link up; Blinking = data transmission |

# Power Supply System

The S5735I-H8U2XN-V2 supports dual redundant DC power input for high availability. It can be powered by an external 54-57V DC power supply or the optional PAC240S56-CN power adapter.

## Power Configurations

| Power Supply Mode | DC external |
| --- | --- |
| Rated Input Voltage | 56V DC |
| Input Voltage Range | 54-57V DC (PoE/PoE+/PoE++); 48V DC (PoE) |
| Maximum Input Current | 6A |
| Redundant Power Supply | 1+1 redundancy supported |
| Typical Power Consumption | 16.4W (48V, 30% traffic) |
| Maximum Power Consumption | 23.3W (without PoE); 270W (full PoE load) |
| Static Power Consumption | 14.2W |

## Heat Dissipation

| Heat Dissipation Mode | Natural heat dissipation (fanless) |
| --- | --- |
| Typical Heat Dissipation | 56 BTU/hour |
| Max Heat Dissipation (no PoE) | 79.5 BTU/hour |
| Max Heat Dissipation (full PoE) | 921.3 BTU/hour |
| Noise Level | Noise-free (no fans) |

# Technical Specifications

## Physical and Environmental

| Dimensions (H x W x D) | 150.0 x 44.0 x 133.0 mm (5.91 x 1.73 x 5.24 in.) |
| --- | --- |
| Maximum Dimensions | 162.0 x 44.0 x 142.0 mm (6.38 x 1.73 x 5.59 in.) |
| Chassis Height | 3.65 U |
| Chassis Material | Metal |
| Weight (without packaging) | 1 kg (2.2 lb) |
| Weight (with packaging) | 1.3 kg (2.9 lb) |
| Operating Temperature (Industrial optical modules, sealed cabinet) | -40°C to +60°C (-40°F to +140°F) |
| Operating Temperature (vented cabinet ≥ 40 LFM) | -40°C to +70°C (-40°F to +158°F) |
| Operating Temperature (blower-equipped cabinet ≥ 200 LFM) | -40°C to +75°C (-40°F to +167°F) |
| Storage Temperature | -40°C to +85°C (-40°F to +185°F) |
| Operating Humidity | 5% RH to 95% RH, non-condensing |
| Operating Altitude | 0-5000 m (0-16404 ft.) |
| Ingress Protection | IP40 |
| Surge Protection (Service Ports) | Common mode: ±6 kV |
| Surge Protection (Power Supply) | ±4 kV common mode, ±2 kV differential mode |

## Performance and Capacity

| Memory | 2 GB |
| --- | --- |
| Flash Memory | 1 GB |
| Console Port | RJ45 |
| USB Port | USB 2.0 Type A |
| DI/DO Port | 2 x 5-Pin Phoenix Connector |
| MTBF | 67.24 years |
| Availability | > 0.99999 |
| Number of Power Slots | 0 (external power) |
| Number of Fan Modules | 0 (fanless) |
| RTC | Not supported |
| RPS Input | Not supported |
| Stacking | Supported (iStack) |
| 1588 PTP | Supported |

# Ordering Information

The following table lists the ordering information for the S5735I-H8U2XN-V2 switch and its accessories.

## Orderable Parts

| Part Number | Description | Remarks |
| --- | --- | --- |
| 98012685 | S5735I-H8U2XN-V2 switch | Without power module |
| 98012685-001 | S5735I-H8U2XN-V2 (variant) | Alternative BOM |
| PAC240S56-CN | 240W Power Adapter | Optional, for PoE support |
| - | 5-Pin DC Power Terminal | Included in accessory kit |
| - | DIN Rail Mounting Kit | Included in accessory kit |
| - | Ground Cable | Included in accessory kit |

Note: The S5735I-H8U2XN-V2 does not include a power module by default. Please order the PAC240S56-CN power adapter separately if needed.

S5735I-H8U2XN-V2

Industrial Ethernet Switch

Hi-Network.com

info@hi-network.com

© 2025 Hi-Network. All rights reserved.