BROCADE GEN 6 FIBRE CHANNEL SWITCH

BR-G620-24-16G-R

______________________________________________________

64-Port Switch with 24 Active Ports and 24 Brocade 16Gb/s Shortwave Multi-mode SFP Modules

2 Tbps Aggregate Bandwidth  |  Gen 6 Fibre Channel  |  1U Rack-mountable

Port-side Exhaust  |  Dual Redundant Hot-swappable Power Supplies

Hi-Network.com

info@hi-network.com

# Product Overview

The Brocade BR-G620-24-16G-R is an enterprise-class Gen 6 Fibre Channel switch designed to meet the demands of hyper-scale virtualization, larger cloud infrastructures, and growing flash-based storage environments. This specific configuration provides 24 active ports with 24 Brocade 16Gb/s shortwave multi-mode SFP modules in a port-side exhaust airflow design, delivering a high-density building block for increased scalability in small to large-scale enterprise infrastructures.

Delivering unmatched performance, exceptional port density, and built-in instrumentation in a space-efficient 1U form factor, the BR-G620-24-16G-R accelerates data access, adapts to evolving requirements, and drives always-on business operations for mission-critical storage area networks (SANs).

## Key Highlights

•  Provides high scalability in an ultra-dense, 1U, 64-port switch platform supporting high-density server virtualization, cloud architectures, and flash-based storage environments

•  Increases performance for demanding workloads across 32 Gbps-capable links with auto-sensing port speeds of 4, 8, 16, and 32 Gbps

•  Enables pay-as-you-grow scalability from 24 to 64 ports for on-demand flexibility via Ports on Demand (PoD) licenses

•  Provides proactive, non-intrusive, real-time monitoring and alerting of storage I/O health and performance with IO Insight integrated network sensors

•  Increases resiliency by automatically discovering and recovering from device or network errors through Brocade Fabric Vision technology

•  Includes 24 Brocade 16Gb/s shortwave multi-mode SFP modules, Web Tools, Zoning, and EGM software authorization

## Product Configuration

| Feature | BR-G620-24-16G-R Specification |
| --- | --- |
| Switch Generation | Gen 6 Fibre Channel |
| Total Ports | 64 ports (48 SFP+ + 4 QSFP) |
| Active Ports | 24 ports with 16Gb/s SW SFP modules |
| Included SFP Modules | 24x Brocade 16Gb/s shortwave multi-mode SFP |
| Port Speeds | Auto-sensing 4, 8, 16, 32 Gbps |
| Airflow Direction | Port-side exhaust (Rear) |
| Form Factor | 1U rack-mountable |
| Software Included | Web Tools, Zoning, EGM license |
| Power Supplies | Dual redundant hot-swappable AC |
| Max Aggregate Bandwidth | 2 Tbps |

# Technical Specifications

| Parameter | Specification |
| --- | --- |
| Aggregate Bandwidth | 2 Tbps |
| Maximum Frame Size | 2,112-byte payload |
| Frame Buffers | 15,360 dynamically allocated |
| Maximum Fabric Latency | < 780 ns (including FEC) |
| Compression Latency | 1 us per node |
| Classes of Service | Class 2, Class 3, Class F (inter-switch frames) |
| Data Traffic Types | Fabric switches supporting unicast |
| Port Types | F_Port, E_Port, M_Port, D_Port (ClearLink Diagnostic), EX_Port |
| Scalability | Full-fabric architecture, maximum 239 switches |
| Certified Maximum | 6,000 active nodes; 56 switches, 19 hops |

## Fibre Channel Performance

| Fibre Channel Speed | Line Rate |
| --- | --- |
| 32 Gbps | 28.05 Gb/s line speed, full duplex |
| 16 Gbps | 14.025 Gb/s line speed, full duplex |
| 10 Gbps | 10.53 Gb/s line speed, full duplex (optionally programmable to fixed) |
| 8 Gbps | 8.5 Gb/s line speed, full duplex |
| 4 Gbps | 4.25 Gb/s line speed, full duplex |
| QSFP Ports (with FOS v8.2.0+) | Auto-sensing 4x32/4x16/4x8/4x4 Gb/s |

# Physical and Environmental Specifications

| Parameter | Specification |
| --- | --- |
| Width | 440.0 mm (17.32 in.) |
| Height | 43.9 mm (1.73 in.) |
| Depth | 355.6 mm (14.00 in.) |
| Weight (without transceivers) | 7.07 kg (15.59 lb) |
| Weight (fully populated) | 8.41 kg (18.54 lb) |
| Operating Temperature | 0°C to 40°C (32°F to 104°F) |
| Operating Humidity | 10% to 85% (non-condensing) |
| Non-operating Temperature | -25°C to 70°C (-13°F to 158°F) |
| Non-operating Humidity | 10% to 90% (non-condensing) |
| Operating Altitude | Up to 3,000 m (9,843 ft) |
| Storage Altitude | Up to 12 km (39,370 ft) |

## Power Specifications

| Parameter | Specification |
| --- | --- |
| Power Supplies | Dual, hot-swappable redundant with integrated cooling fans |
| AC Input | 90V to 264V ~, maximum input current 3.5A |
| Input Line Frequency | 47 Hz to 63 Hz |
| Power Consumption (empty chassis) | 85W (no optics) |
| Power Consumption (fully populated) | Maximum 205W (48x 32G SFP+ + 4x 4x32G QSFP) |
| Frame-based ISL Trunking | 256 Gb/s (optional) |

## Port Configuration Details

The BR-G620-24-16G-R ships with 24 active Fibre Channel ports enabled. The switch chassis supports a maximum of 64 ports (48 SFP+ ports and 4 QSFP ports). Additional ports are enabled through 12-port SFP+ increments via Ports on Demand (PoD) licenses and through one 4-port QSFP PoD, providing 16-port increments through a Q-Flex license.

Port types supported: E_Ports, F_Ports, M_Ports, D_Ports (ClearLink Diagnostic Port), and EX_Ports. In Brocade Access Gateway mode, the switch supports 40 SFP+ F_Ports and 8 SFP+ N_Ports.

# Scalability and Management

| Parameter | Capability |
| --- | --- |
| Fabric Architecture | Full-fabric, maximum 239 switches |
| Access Gateway | Supports Brocade Access Gateway mode |
| FICON Support | Supported |
| ISL Trunking | Up to 8x 32 Gbps SFP+ per trunk; up to 2x 128 Gbps QSFP per trunk |
| Port Expansion | 12-port SFP+ increments via PoD licenses; 4-port QSFP via Q-Flex license |
| Management Interfaces | RJ-45 Ethernet, in-band FC, serial RJ-45, USB |
| Vibration (Operating) | 0.5 g sine, 0.4 grms random, 5 to 500 Hz |
| Vibration (Non-operating) | 2.0 g sine, 1.1 grms random, 5 to 500 Hz |

## Management and Monitoring

•  Management Access: 10/100/1000 Mbps Ethernet (RJ-45), in-band over Fibre Channel, serial port (RJ-45), and one USB port

•  USB Port: One USB port for system log file downloads or firmware upgrades

•  ISL Trunking: Frame-based trunking with up to eight 32 Gbps SFP+ ports per ISL trunk or up to two 128 Gbps QSFP ports per ISL trunk; exchange-based load balancing across ISLs with DPS included in Brocade Fabric OS

•  Brocade Fabric Vision Technology: Simplifies administration, quickly resolves problems, increases uptime, and reduces costs

•  VM Insight: Enables Virtual Machine (VM) visibility in a storage fabric to monitor and optimize VM performance

•  IO Insight: Industry's first integrated network sensors for proactive, real-time monitoring and alerting of storage I/O health and performance

## Standards Compliance

Certifications: CE, SA, KCC. The Brocade G620 switch meets RoHS Directive requirements. The switch supports Brocade Fabric OS and is certified for up to 6,000 active nodes in a single fabric with up to 56 switches and 19 hops.

## Ordering Information

| Item | Description |
| --- | --- |
| Base Model | BR-G620-24-16G-R: 64-port switch, 24-port activation, 24x 16Gb/s SW SFP, port-side exhaust |
| 12-Port Upgrade Kit | XBR-SMED12POD-16G: 12-port PoD with 12x 16Gb/s SW SFP modules |
| 8-Port Upgrade Kit | XBR-SMED8PTPOD-16G: 8-port PoD with 8x 16Gb/s SW SFP modules |
| Rack Mount Kit | XBR-R000296: Rack mount kit for 1U installation |
| USB Flash Drive | XBR-USB-4GB: 4GB USB flash drive for firmware and logs |

Hi-Network.com

Your Trusted Network Equipment Partner

______________________________

Email: info@hi-network.com

Web: www.Hi-Network.com

Brocade BR-G620-24-16G-R Technical Datasheet

All trademarks are property of their respective owners. Specifications subject to change without notice.