FIBRE CHANNEL SWITCH

BR-G610-8-16G-0

Brocade G610 8-Port Fibre Channel Switch

24-Port Switch with 8-Port Activation & 8 x 16 Gbps SWL SFP+ Transceivers

www.Hi-Network.com

info@hi-network.com

# Product Overview

The Brocade G610 switch (Model: BR-G610-8-16G-0) is an affordable, flash-ready Fibre Channel switch that delivers enterprise-class availability for small to mid-sized businesses. Purpose-built for modern storage environments, it provides high-throughput, low-latency connectivity with the flexibility to scale on demand.

This configuration includes 8 active ports with 8 x 16 Gbps Shortwave (SWL) SFP+ transceivers pre-installed. The switch supports pay-as-you-grow expansion from 8 to 24 ports via Ports on Demand (PoD) licenses, enabling organizations to start small and scale without disruption.

## Key Highlights

•  Affordable Gen 6 Fibre Channel switch with 8 activated ports and 8 x 16G SWL SFP+ transceivers

•  Flash-ready performance — seamlessly transition to next-generation NVMe flash arrays

•  Scales on demand from 8 to 24 ports via Ports on Demand (PoD) licenses

•  Enterprise-class reliability with Brocade Fabric Vision technology

•  Simple three-step installation with point-and-click user interface

•  768 Gb/s aggregate bandwidth with auto-sensing 4/8/16/32G port speeds

## Package Contents

•  Brocade G610 Switch with 8 activated ports

•  8 x Brocade 16 Gbps SWL SFP+ transceivers (pre-installed)

•  1 x Fixed AC power supply with integrated cooling fans

•  Rack mount kit

•  Brocade Web Tools, Zoning, and EGM software license

# Technical Specifications

## System Architecture

| Fibre Channel Ports | 24 ports total, 8 activated (expandable to 16/24 via PoD) |
| --- | --- |
| Port Speeds | Auto-sensing 4, 8, 16, and 32 Gbps |
| Aggregate Bandwidth | 768 Gb/s end-to-end full duplex |
| Maximum Fabric Latency | < 780 ns (including FEC) |
| Maximum Frame Size | 2,112-byte payload |
| Frame Buffers | 2,000 dynamically allocated |
| Classes of Service | Class 2, Class 3, Class F (inter-switch frames) |
| Port Types | F_Port, E_Port, M_Port, D_Port (ClearLink Diagnostic) |
| Access Gateway Mode | F_Port and NPIV-enabled N_Port |
| Scalability | Full-fabric, maximum 239 switches |
| Certified Maximum | 6,000 active nodes; 56 switches, 19 hops |

## ISL Trunking

| Frame-Based Trunking | Up to 8 x 32G SFP+ ports per ISL trunk |
| --- | --- |
| Maximum ISL Bandwidth | 256 Gb/s per ISL trunk |
| Load Balancing | Exchange-based Dynamic Path Selection (DPS) |

## Physical Specifications

| Dimensions (W x H x D) | 428.8 x 42.9 x 306.6 mm (16.88 x 1.69 x 12.07 in.) |
| --- | --- |
| Weight (without transceivers) | 4.20 kg (9.26 lb) |
| Weight (fully populated) | 4.80 kg (10.58 lb) |
| Rack Units | 1U |
| Airflow | Port-side exhaust |

# Features & Management

## Advanced Features

•  ISL Trunking — Frame-based trunking with up to 256 Gb/s per ISL trunk

•  Dynamic Path Selection (DPS) — Automatic routing to the most efficient path

•  Fabric Vision — Proactive VM health monitoring and diagnostics

•  ClearLink Diagnostics — Optics and cable diagnostics with electrical/optical loopback

•  Forward Error Correction (FEC) — Recovery from bit errors on device connections and ISLs

•  Credit Loss Recovery — Overcomes performance degradation from buffer credit loss

•  Flow Mirroring — Non-disruptive capture of specific application flows for analysis

•  Built-in Flow Generator — Pretest and validate infrastructure before deployment

## Fabric Services

•  Brocade Advanced Zoning (default, Port/WWN, peer, target-driven zoning)

•  Dynamic Fabric Provisioning (DFP)

•  Fabric Performance Impact (FPI) Monitoring

•  Slow Drain Device Quarantine

•  Monitoring and Alerting Policy Suite (MAPS)

•  Flow Vision

•  VM Insight

## Management & Security

| Management Access | 10/100/1000 Mb/s Ethernet (RJ-45), in-band FC, Serial (RJ-45), USB |
| --- | --- |
| Management Software | Brocade SANnav, Web Tools, CLI, RESTful API |
| Protocols | HTTP/HTTPS, SNMP v1/v3, SSH, SMI-S |
| Authentication | DH-CHAP, FCAP, RADIUS, TACACS+, LDAP, RBAC |
| Security | IPsec, IP filtering, Port Binding, SSH v2, SSL, SFTP, SCP |
| USB Port | 1 x USB for log file download and firmware upgrades |

# Environmental & Power

| Power Supply | Single, fixed power supply with 4 integrated cooling fans |
| --- | --- |
| AC Input | 90 V to 264 V ~ 3.5A |
| Max Input Current | 2.2A |
| Power Consumption (Max) | 76.52W (all 24 ports with 32G optics) |
| Power Consumption (Idle) | 55.83W (optics loaded, not initialized) |
| Heat Dissipation | 215 BTU/hr (24 ports at max) |
| Operating Temperature | 0°C to 40°C (32°F to 104°F) |
| Operating Humidity | 10% to 85% (non-condensing) |
| Non-Operating Temp | -25°C to 70°C (-13°F to 158°F) |
| Operating Altitude | Up to 3,000 m (9,842 ft) |
| Operating Vibration | 0.5g sine, 0.4 grms random, 5 Hz to 500 Hz |
| Operating Shock | Up to 20 G, 6 ms half-sine |

# Ordering Information

| Part Number | BR-G610-8-16G-0 |
| --- | --- |
| Description | Brocade G610, 24-port switch, 8-port activation with 8 x 16 Gbps SWL SFP+ transceivers |
| Generation | Gen 6 Fibre Channel |
| Lifecycle Status | Active |
| Certifications | CE, SA, KCC |
| Country of Origin | Czech Republic |

## Upgrade Options

•  BR-SMED8PTPOD-01 — 8-port upgrade kit with 8 x 16G SWL SFP+ transceivers

•  BR-SMED8PTPOD-16G — 8-port upgrade with 8 x 16 Gbps SWL SFP+ modules

© 2025 Hi-Network.com. All product names, logos, and brands are property of their respective owners. Specifications are subject to change without notice.